# GT6300

# SECTION 08 51 13

ALUMINUM WINDOWS

GRAHAM ARCHITECTURAL PRODUCTS

FIXED WINDOW

SERIES GT6300

AW-PG100 (AAMA/WDMA/CSA 101/I.S.2/A440-08)

February, 2026

(The following information is based on current information at the time of publication

Check www.grahamwindows.com for most recent product ratings)

PART 1 GENERAL

  - SUMMARY

- Related Documents:  Conditions of the Contract, Division 1 - General Requirements, and Drawings apply to Work of this Section.

*********************************************************************************************************

Edit this paragraph to briefly describe the contents of the section.  After editing section, refer back to this paragraph to verify no conflicts exist.

*********************************************************************************************************

- Section Includes:

  - Aluminum Prime Windows

    - Type:___________ [Choose all that apply:  Fixed]

    - Category: Architectural (AW)

    - Designation:___________ [Choose all that apply: FW-AW100]

- Related Sections

  - Drawings, General and Supplementary Conditions of the Contract, Division 1 and the following Specification Sections, apply to this Section.

  - Section 01 41 00 – Regulatory Requirements

  - Section 01 43 00 – Quality Assurance

  - Section 07 92 00 – Joint Sealants

  - Section 08 51 13 – Aluminum  Windows

  - Section 08 81 00 – Glass Glazing

  - REFERENCES

(Note:  Delete all reference standards that are not actually required and add any additional standards required by the municipality and/or state where the project is located.  The Contracting Requirements or Division 1, Section 01420 – References, may establish the edition date of standards not otherwise indicated.  Division 1 may include full names and addresses of the organizations whose standards are referenced.)

- American Architectural Manufacturers Association (AAMA):

  - AAMA/WDMA/CSA 101/I.S.2/A440–Standard Specifications for Windows, Doors, and Skylights

  - AAMA 1503 – Voluntary Test Method for Thermal Transmittance and Condensation Resistance of Windows, Doors and Glazed Wall Sections

- American National Standards Institute (ANSI)

  - ANSI Z97.1 - American National Standard for Safety Glazing Materials Used in Buildings - Safety Performance Specifications and Methods of Test.

- ASTM International

  - ASTM E283 - Standard Test Method for Determining Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen.

  - ASTM E330 - Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights and Curtain Walls by Uniform Static Air Pressure Difference.

  - ASTM E331 - Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors, and Curtain Walls by Uniform Static Air Pressure Difference.

  - ASTM E2188 - Standard Test Method for Insulating Glass Unit Performance.

  - ASTM E2189 - Standard Test Method for Testing Resistance to Fogging in Insulating Glass Units.

  - ASTM E2190 - Standard Specification for Insulating Glass Unit Performance and Evaluation.

- Consumer Product Safety Commission (CPSC)

  - CPSC 16 CFR 1201 - Safety Standard for Architectural Glazing Materials.

- Glass Association of North America (GANA)

  - Glazing Manual (current edition)

  - SYSTEM DESCRIPTION

- Test Procedures and Performance

  - Specifications for Windows, Doors and Unit Skylights: AAMA 101.

  - Air Infiltration Test: ASTM E 283, at 6.24 psf static air pressure differential.  Air infiltration shall not exceed 0.10 CFM per sq. ft.

  - Water Resistance Test:  ASTM E 331, no water leakage at 15 psf static air pressure differential.

  - Uniform Load Deflection Test: ASTM E 330, at static air pressure of +/- 100 psf.  No member shall deflect more than 1/175 of its span.

  - Uniform Load Structural Test: ASTM E 330, at static air pressure difference of +/- 150 psf.

  - Condensation Resistance Test: AAMA 1503.1, CRF shall be not less than 55.

  - Thermal Transmittance Test: AAMA 1503.1, U-Value shall not exceed .60 BTU/HR/SQ.FT/°F.

  - Acoustical Test: ASTM E 90 / ASTM E 413, reference 2.02, D. for acoustical glass type, or Section 08 81 00 [Delete 1.03, A., 9. if there are no sound abatement requirements.]

  - Human Impact test: AAMA 501.8, Tested to 2000ft-lbs

  - Provides Ligature Resistance

    - Accepted by NYS OMH

  - SUBMITTALS

- Provide submittals in a timely manner to meet required construction completion schedule and in accordance with specifications.

  - Shop Drawings

    - Shop drawings will be prepared by the window manufacturer. Shop drawings prepared by window distributor, installer, representative/dealer of outside drafting firm are not acceptable.

    - Show components complete with dimensions, material and details of anchoring and fastening.

    - Show finishes, sealants and other information indicating compliance with specifications.

    - Submit test report per 1.03 SYSTEM DESCRIPTION A. Test Procedures and Performance.

  - Samples

    - Components: submit samples of anchors, fasteners, hardware, assembled corner sections and other materials and components if requested by architect.

    - Finish: submit full range color samples for approval by architect.

  - Warranties: submit written copies in accordance with - 1.08 WARRANTIES

  - DELIVERY, STORAGE AND HANDLING

- Protect materials from damage before installation per instructions and in accordance with specifications.

  - QUALITY ASSURANCE

- Installer Qualifications: An experienced installer that has completed the same or similar projects in size and scope.

- Source Limitations: Obtain aluminum windows from single source manufacturer.

- In-House Testing: Conduct air and/or water testing of 2% windows prior to shipping.

- Detailed documentation on in-house testing is available upon request.

  - PROJECT / SITE CONDITIONS

- Ensure ambient and surface temperatures and joint conditions are suitable for installation of materials.

  - WARRANTY

- Window System

  - Qualified window manufacturer, with proven financial responsibility and years of experience of at least the length of the warranty period shall provide written ____ [2, 5, or 10] year warranty against defects in materials and                         workmanship.

PART 2 PRODUCTS

  - MANUFACTURERS

  - Basis of Design: Series GT6300 profile fixed window as manufactured by Graham Architectural Products, York, PA.

  - MATERIALS

- Aluminum: 6063-T5 alloy shall have 0.125” wall thickness.

  - Extrusions: comply with ASTM B 221.  Extrusion tolerances shall meet ANSI H35.2.

  - Sheet: comply with ASTM B 209.

  - Frame Depth:  4 1/8".

  - Thermal Barrier:  Dual Strut Glass Reinforced Polyamide Strips

- Hardware: material shall be corrosion resistant and compatible with aluminum.  Hardware must prove its strength and suitability by being installed on units that are tested in accordance with specifications.

  - Fasteners: provide non-magnetic stainless steel screws, epoxy adhesives, or other material warranted by the manufacturer.

  - Operating Hardware: Manufacturers standard

- Sealants: color of exposed sealants shall be compatible with adjacent window materials.  Comply with AAMA 803.3.

- Glazing: windows shall be factory glazed unless too large or unsafe for handling.

  - Glass: provide in accordance with Section 08 81 00.

  - Glazing Materials: units shall be exterior wet glazed using silicone cap beads, setting blocks, edge blocks and accessories as recommended by and in accordance with GANA Glazing Manual.

- Access Panels: shall be ____________ [Choose the one that applies: hinged, lift-off] mitered, epoxied, mechanically crimped over gussets and sealed to form a hairline joint. [Delete 2.02 E. if access panels are not used.]

- Blinds: shall be 5/8” or 1” with pull cord, slip clutch and tilt control. [Delete 2.02 F. if blinds are not used.]

  - Aluminum surfaces shall be coated with baked enamel.  Blind color shall be: _______________ [Choose those that apply: Reference Blind Color Selection chart].

  - FABRICATION

- Frames: shall be machined, mechanically fastened and sealed to form a watertight joint.

- Component Forming: all aluminum components shall be formed, free of scratches and burrs, before application of finish.

  - FINISHES

- Cover all exposed areas of aluminum windows and components. Overall finish shall be____________________. [Choose one that applies: Clear Anodized, Color Anodized,  70% Polyvinylidene Fluoride.]

  - [Clear Anodized]

    - Type: Architectural Class I clear anodizing

    - AAMA Specification: Comply with AAMA 611

    - Aluminum Association Designation: AA-M10-C22-A41

    - Color:  Clear 215-R1

  - [Color Anodized]

    - Type: Architectural Class I for color anodizing

    - AAMA Specification: Comply with AAMA 611

    - Aluminum Association Designation: AA-M10-C22-A44

    - Color: __________ [ Champagne bronze, light bronze, medium bronze, dark bronze, or black]

  - [70% Polyvinylidene Fluoride]

    - Type: high performance baked-on organic coating

    - AAMA Specification: Comply with AAMA 2605

    - Color: __________

PART 3 EXECUTION

  - EXAMINATION

- Verify that openings are dimensionally within allowable tolerances, plumb, level, clean, provide a solid anchoring surface and are in accordance with approved shop drawings.

  - INSTALLATION

  - Install windows with skilled tradesman in accordance with approved shop drawings and specifications.

  - Unfinished aluminum shall be insulated from direct contact with steel, masonry concrete, and non-compatible materials by bituminous paint, zinc chromate primer or other suitable insulating material.

  - Install vapor retarder/air barrier in accordance specifications between window perimeter and adjoining collateral materials and existing wall barriers to assure continuity.

  - Plumb window faces in a single plane for each wall plane.  Erect square and true.  Anchor to maintain position when subjected to normal thermal and building movement, seismic forces and specified wind loads.

  - Apply sealants at joints and intersections and at opening perimeters in accordance with approved shop drawings and Section 07 93 13 to provide watertight installation.

  - FIELD QUALITY CONTROL

- Conduct on-site air and water infiltration tests in accordance with AAMA 502, ASTM E 783, ASTM E 1105, and with architect and window manufacturer's representative present.  Architect will select units to be tested.  Air infiltration shall not exceed 1.5 x air infiltration amount specified for laboratory testing.

- Tested units not meeting specified requirements and units having similar deficiencies shall be corrected at no cost to owner.

- Cost for successful tests shall be paid by owner.  Unsuccessful tests shall be paid by contractor.

- Testing shall be by agency acceptable to architect and window manufacturer and employed by contractor.

  - CLEANING

- After installation and testing, windows and glazing shall be inspected, adjusted, and left clean and free of labels and dirt.  Protect finished installation against damage.

- Final cleaning of anodized finish shall be in accordance with AAMA 609.1; painted finish shall be in accordance with AAMA 610.1.

END OF SECTION