SECTION 08 12 16

ALUMINUM DOORS AND FRAMES

- GRAHAM ARCHITECTURAL PRODUCTS

- SERIES GT7700

- TYPE: ALUMINUM TERRACE DOOR (Outswing)

- AW-PG60 (AAMA/NWWDA/CSA 101/I.S.2/A440-08)

- September, 2016

- (The following information is based on current information at the time of publication

- Check www.grahamwindows.com for most recent product ratings)

# PART 1 - GENERAL

- SUMMARY

  - The Conditions of the Contract, and all Sections of Division 1, are hereby made a part of this Section.

  - Section Includes:  Factory glazed thermally broken, aluminum terrace doors complete with necessary mullions, panning, trim, expanders, operating hardware, installation hardware and all other accessories as required.

  - Related Sections:

    - Division 7 Section “Joint Sealants.”

    - Division 8 Section “Glass and Glazing.”

  - Coordinate work with that of all construction contractors affecting or affected by work of this Contract.  Cooperate with such contractors to assure the steady progress of the Work.

  - Conduct field testing of windows when specified in Division 1 by an independent AAMA accredited lab using AAMA 502-08 field test procedures.

- SYSTEM DESCRIPTION

  - General:  In addition to requirements shown or specified comply with applicable provisions of AAMA/NWWDA/CSA 101/I.S.2/A440-08 for design, materials, fabrication and installation of component parts.

  - Door Replacement Requirements:

    - Work Included:  Provide labor, materials and equipment necessary to complete the work of the Replacement Terrace Door Contract, and without limiting the generality thereof include:

    - Removal of existing sash, fixed glazing, frames and other accessories as required by the proposed replacement system.

    - Removal of other existing work as required for the proper installation and operation of the new units.

    - Removal from site and legal disposal of all removed materials, debris, packaging, banding and all other surplus materials and equipment.

    - Insulated panels and frames as required in selected transoms and other locations.

    - Treated wood blocking, fillers and nailers as required for secure installation.  Bidders shall survey conditions of existing sills and jambs prior to bidding.  Contractor shall be responsible for providing new blocking for portions of same that are deteriorated.

    - Fiberglass insulation between door frames and adjacent construction.Sealing of all joints within each door assembly.

    - Sealing of entire exterior perimeter of door units after installation.

    - Field observations and measurements of existing openings and conditions.

    - Furnishing and delivering of extra materials as specified.

  - New Construction:

    - Coordinate with the Architect and the general contractor that the opening dimensions will be maintained in accordance with the documents.

    - It is the responsibility of the sub-contractor to allow proper clearance for the type of installation in accordance with the manufacturer's standard procedures.

  - Design Requirements:

    - Manufacturer/subcontractor is responsible for designing system, including installation instructions and necessary modifications to meet specified requirements and maintain visual design concepts.

    - Requirements shown by details are intended to establish basic dimension of unit, sight lines and profiles of members.

    - Provide assemblies free from rattles, wind whistles and noise due to thermal and structural movement and wind pressure.

    - Installation instructions are to take into account specified site peculiarities and expansion and contraction movements so there is no possibility of loosening, weakening or fracturing connection between units and building structure or between units themselves.

    - Provide for expansion and contraction due to structural movement without detriment to appearance or performance.

    - Evacuate water without infiltration to interior from exterior face of wall, water entering joints, and condensation occurring within doors, by drain holes and gutters of adequate size or other acceptable method.

    - Provide concealed fastening wherever possible.

  - Performance Requirements:  Requirements for aluminum windows, terminology and standards of performance, and fabrication and workmanship are those specified and recommended in AAMA/NWWDA/CSA 101/I.S.2/A440-08 and applicable general recommendations published by AAMA.  Conform to more stringent of specified AAMA standards and following:

    - Air Infiltration Test: Not exceed 0.10 cubic feet per minute per foot of crack length when tested at a pressure of 6.2 psf. Perform tests in accordance with ASTM E 283 with the sash in a closed and locked position.

    - Water Resistance Test:  Subject door unit to a water resistance test in accordance with ASTM E 331 with no water passing the interior face of the door frame and no leakage as defined in the test method.  Mount the glazed unit in its vertical position continuously supported around the perimeter and the door sash placed in the fully closed and locked position. When a static pressure of 12 pounds per square foot has been stabilized, apply five gallons of water per square foot of window area to the exterior face of the unit for a period of 15 minutes. [Contact factory for water performance when using the low profile sill]

    - Uniform Load Deflection Test: ASTM E 330 at 60 pounds per square foot: No member deflection more than 1/175 of its span.  Maintain test load for a period of 10 seconds resulting in no glass breakage, permanent damage of fasteners, hardware parts, support arms, actuating mechanisms or any other damage causing the window to be inoperable.

    - Uniform Load Structural Test: Apply a minimum exterior and interior uniform load of 90 pounds per square foot to the entire outside surface of the test unit.  Maintain this test load for a period of 10 seconds. Results: No glass breakage, permanent damage of fasteners, hardware parts, support arms, actuating mechanisms, or any other damage causing the door to be inoperable. And no permanent deformation of any frame or vent member in excess of 0.2 percent of its span.

    - Life Cycle Test: Per AAMA 101 and AAMA 910, provide proof that the product meets the criteria including passing air and water tests at the conclusion of the cycle tests.

    - Operation Cycling Performance: No damage after 25,000 cycles when tested in accordance with AAMA 920

- NOTE:  U-Value and Solar Heat Gain Coefficient will varry with different glazing combinations.  Overall unit thickness, glass thickness, type and number of low-E coatings, tints, spacers and airspace will impact the performance.

- Frame depth selection will also affect U-values. The deeper frame has a larger thermal seperation and will perform better.

- Contact manufacturer to review your your specific project requirements and modify in the following three paragraphs.

    - “U” Factor:  Perform computer simulation in accordance with NFRC 100-2004 on a 1000 mm x 2000 mm maximum size window resulting in a whole window U-Factor of no more than _____ using soft coat low-E insulating glass, argon and a warm edge spacer.

    - Solar Heat Gain Coefficient: Perform computer simulation in accordance with NFRC 200-2004 on a 1000 mm x 2000 mm maximum size window resulting in a whole window SHGC of no more than _____ using soft coat low-E insulating glass, argon and a warm edge spacer.

    - Condensation Resistance: Perform computer simulation in accordance with NFRC 500-2004 on a 1000 mm x 2000 mm maximum size window resulting in a whole window CR of no less than _____ using soft coat low-E insulating glass, argon and a warm edge spacer.

    - Testing:  Where manufacturer's standard window units comply with requirements and have been tested in accordance with specified AAMA/NWWDA/CSA 101/I.S.2/A440-08 and NFRC tests, provide certification by AAMA certified independent laboratory showing compliance with such tests.  Submit copy of the test report signed by the independent laboratory.

- SUBMITTALS

  - Product Data:  Submit manufacturer's specifications, recommendations and standard details for aluminum terrace doors.

  - Shop Drawings:  Submit shop drawings, including location floor plans or exterior wall elevations showing all door openings, typical unit elevations at 1/4 inch scale, and half size detail sections of every typical composite member.  Show anchors, hardware, operators and other components as appropriate if not included in manufacturer's standard data.  Include glazing details and standards for factory glazed units.

  - Samples:

    - Submit one sample of each required aluminum finish, on 3 x 3 inch long sections of extrusion shapes or aluminum sheets as required for door units.

    - Submit additional samples, if and as directed by Architect, to show fabrication techniques, workmanship of component parts, and design of hardware and other exposed auxiliary items.

  - Certifications:  Submit certified test laboratory reports by independent laboratory substantiating performance of system.  Include other supportive data as required.

- PRODUCT DELIVERY, STORAGE AND HANDLING

  - Store and handle doors, mullions, panels, hardware and all pertinent items in strict compliance with the manufacturer's instructions.

  - Protect units adequately against damage from the elements, construction activities and other hazards before, during and after installation.

- WARRANTY

  - Manufacturer's Warrantees:  Submit written warrantees from manufacturer for the following:

    - Terrace Doors: Doors furnished are certified as fully warranted against any defects in material or workmanship under normal use and service for a period of ten (10) years from date of fabrication.

- Note:  Anodized finishes are not warranted outside of the manufacturer’s standard 1 year warranty.  Retain the following paragraph only if baked on or pigmented organic finishes (Kynar type coatings) are used.  Refer to Finishes Article in this specification for description of AAMA standards.

    - Finish: The pigmented organic finishes on exposed surfaces of doors and component parts (such as trim, mullions and the like) are certified as complying fully with requirements of AAMA 2603 [2604] [2605] for pigmented organic coating and fully warranted against chipping, peeling, cracking or blistering for a period of five (5) years from date of installation. [Warrant finishes meeting AAMA 2604 and 2605 against fading beyond AAMA standards.]

    - Insulated Glass: Warranted from visual obstruction due to internal moisture for a period of five (5) years; [ten (10) years]; [twenty (20) years].

- MAINTENANCE MATERIAL

- Note:  Please specify below additional material desired and quantity or percentage of additional material.

  - Option for additional materials:  Furnish and deliver to Owner, at the project location, extra material for maintenance/replacement as listed.

    - Touch-up Paint:

    - Locks:

    - Glass Units for Doors:

    - ________________________________________.

    - ________________________________________.

  - Upon delivery, obtain signed receipt from Owner's representative.  Include copy of receipt with submittals required at time of Substantial Completion.

PART 2 - PRODUCTS

- MANUFACTURERS

  - Basis of Design: Series GT7700 Terrace Door (Outswing) as manufactured by Graham Architectural Products.

  - Thermal Barrier: Provide a continuously extruded, multi-directional 25% glass fiber reinforced 6/6 polyamide nylon (Strut). Aluminum window framing members separated up to a maximum of 3” with a locking mechanical connection to the Thermal Strut by properly knurling the aluminum cavity and crimping the Strut into place to create a composite thermal barrier assembly. Structural performance values of the Thermal Barrier assembly to meet specific product/project design criteria or at a minimum certified testing criteria and procedures as described by the AAMA TIR-A8 performance standards. Other thermal barrier assemblies such as rolled-in PVC, single directional glass fiber-reinforced polyamides, or pour-and-debridged polyurethane systems will not be accepted

  - Pre-Bid Qualifications:

    - All bids must be based on pre-qualified products.  To qualify, the bidder must furnish one complete typical project size terrace door unit 10 days prior to the time set for bids.  Accompanying the sample will be certified test reports from an accredited AAMA Laboratory verifying that the performance of the product meets or exceeds the AW60 classification.

    - This sample must be a true and accurate representation of the door the bid is based on with the finish being the only exception.  No verbal approvals will be given.  Each submitter will be notified in writing of acceptance or rejection.

    - The manufacturer must verify that it has been engaged in the manufacturing of the product in their production facility for a period of five (5) years.

    - Maintenance manuals accompany the product sample being submitted for approval.

    - Sight lines to match the base product specified.

    - The qualified bidder must verify that the bidder has been involved with the installation of this type of product in a minimum of 5 projects of similar scope and quality.

- MATERIALS

  - Aluminum Extrusions:  Alloy and temper recommended by door manufacturer for strength, corrosion resistance and application of required finish, but not less than 22,000 psi ultimate tensile strength, a yield of 16,000 psi. Comply with ASTM B 221.

  - Fasteners:  Aluminum, stainless steel, or other materials warranted by manufacturer to be non-corrosive and compatible with aluminum door members, trim, hardware, anchors and other components of door units.

    - Do not use exposed fasteners on exterior except where unavoidable for application of hardware.  Match finish of adjoining metal.

    - Provide non-magnetic stainless steel, Phillips oval-head machine screws for exposed fasteners, where required, or special tamper-proof fasteners.

    - Locate fasteners so as not to disturb the thermal barrier construction of doors.

  - Anchors, Clips And Door Accessories:  Depending on strength and corrosion-inhibiting requirements, fabricate units of aluminum, non-magnetic stainless steel or hot-dip zinc coated steel or iron complying with ASTM A 123.

  - Compression Glazing Strips and Weatherstripping:  At manufacturer's option, provide EPDM gaskets complying with ASTM D 2000 Designation.

  - Sealant:

    - Unless otherwise indicated for sealants required within fabricated door units, provide elastomeric type as recommended by door manufacturer for joint size and movement, to remain permanently elastic, non-shrinking and non-migrating.

    - Refer to Division 7 for perimeter sealants between door units and surrounding construction.

- DOOR TYPES (OPERATION)

- NOTE: The following paragraphs define operating arrangements for types of sash required in door units and specify minimum provisions for each type.  Drawings show which panels of each door unit are operable sash and which are fixed.

  - General: Except as otherwise indicated, provide door units complying with requirements of AAMA Classification "AW" type windows.  Terrace doors for this project will be rated a minimum of AW60 for full size test units to withstand a design pressure of 60 psf minimum.

  - Architectural Terrace Doors (ATD):

    - Frame Depth:  Minimum 2.750, 3.375 or 3.750 inches. (The frame depth is dependent on the thermal strut dimension selected.  Consult Graham for assistance if required)

    - Door leaf: nominal 2.50” deep

- FABRICATION AND ACCESSORIES

  - General:  Provide manufacturer's standard fabrication and accessories which comply with specifications.  Include complete system for assembly of components and anchorage of door units and provide complete pre-glazing at the factory.

  - Terrace Door Material:

    - Door Members:  Aluminum.

    - Secondary Members (friction tabs, shoes, weatherstripping guides, etc.): Aluminum or a material compatible with aluminum.

    - Standard wall thickness tolerance: In accordance with the Aluminum Association.

  - Master Frame: Not less than 2.75 inches in depth.

Door leaf: nominal 2.50” deep

  - Thermal Barrier:   Provide continuous dual glass fiber reinforced 6/6 polyamide nylon struts around the entire perimeter of the frame and all sash.  Thermal barrier shall not be bridged by  metal conductors at any point.

  - Hardware:

    - Material: Aluminum, stainless steel or other non-corrosive materials compatible with aluminum for hardware having component parts which are exposed.  Cadmium or zinc-plated steel where used must be in accordance with ASTM Specification B 766 or B 633.

    - Provide concealed locking system that automatically activates three locking points on the  lock side jamb one locking point at the head and one locking point at the sill.

    - Provide thumb turn deadbolt lock that operates from the interior.  [Optional exterior keyed cylinder is available]

    - Lever: Mount on door interior and exterior.

      - Single acting handles to provide means to engage the locks in multiple locations on the door.

      - Handle style options:

        - Savio: Manon

        - Optional handles are available.  Please contact your representative for availability.

      - Handle Finish options:

        - Savio:  RAL Powder Coated Finish.  Standard colors are White, Black, Silver and Cream

        - Custom colors and finishes are available.  Please contact your representative for availability.

- Note:  Please indicate the finish required on the hinge below and adjust paragraph 3.  While custom colors are available, there is a cost and lead time impact.  Please check with your representative on the cost and lead time impact for custom color handles and hinges.

        - Summa Hinge:  2 wing adjustable aluminum hinge, 2 quick clamping counterplates, 4 socket head cap screws M6x12 and 1 grub screw M5x6 in stainless steel, 1 stainless steel 10 mm pin preassembled on black galvanized zamak cam bushing device, 2 nylon caps, 1 self-lubricating bushing in nylon, 1 adjustment grub screw in zamak. Weight capacity: 2 hinges 220 lbs, 3 hinges 264 lbs.   Hinge to be painted in manufacturers standard colors [White, Black, Silver, Cream]

***** OR *****

        - NOTE: The following paragraphs define operating arrangements for types of sash required in door units and specify minimum provisions for each type.  Drawings show which panels of each door unit are operable sash and which are fix[Hinge to be painted in a custom color]  NOTE: The following paragraphs define operating arrangements for types of sash required in door units and specify minimum provisions for each type.  Drawings show which panels of each door unit are operable sash and which are fix

  - Construction:

    - Frame and Door corners shall be double keyed, staked and sealed.

    - Water control: 1-1/4” wide by 5/8” high weep slot shall be located 3”from each end of the sill for drainage.

  - Mullions - Other structural members:  When mullion units occur, whether they are joined by integral mullions, independent mullions or by a combination of frame members, the resulting members must be capable of withstanding the load outlined under Uniform Load specified load requirements, without deflecting more than 1/175th of its span.  When independent or integral mullions are used to join doors, the mullions shall contain a thermal barrier as specified.  Evidence of compliance may be by mathematical calculations.

  - Glazing:

    - Pre-glaze all units (except as required for installation) at the factory with insulated glass as follows:

      - Typical Insulated Glass: Overall thickness of 1 inch with two lites of 1/8 inch or 3/16 inch or 1/4 inch as size and loading require.

        - Primary Sealant: Polyisobutylene applied to the edge of the spacer.

        - Secondary Sealant: Silicone.

        - Air Spacer: Continuous metal spacer with formed corners and an in-line connector, containing desiccant.

    - Glaze units to allow for glass replacement without the use of special tools. Sash and frame shall be dry glazed.  Interior of sash and frame shall contain a continuous preset EPDM gasket.  Exterior of frame and sash shall contain a removable EPDM wedge gasket.

    - Provide setting blocks on all four sides of the glass at 1/4 point.

    - Use of tapes, vinyl, or other types of sealant not specified is not allowed.

  - Exterior Grids: Hollow extruded aluminum or flat bar, finish to match the window system.  Trapezoidal profile 0.750 inches [1.250 inches] x 0.312 inches deep, or as shown on plans, with a 45 degree slope to match perimeter sash frame. Machine and mechanically fasten the intersections of the muntin grids.  Fasten grid to sloped perimeter frame at each contact point.  Two piece snap grids are not permitted as a substitute.

  - In-between Glass Muntin (Internal Grid):  [5/8 or 1] inch profile muntin suspended in-between the two lights of glass, so there is no contact between the glass and the muntin.  Mechanically join all intersections of the muntin and mechanically attach muntin to the spacers in the glass unit

- TRIMS AND MULLION COVERS

  - Exterior mullion covers: Extruded aluminum shape to provide rigidity, no less than nominal 0.062 inch wall thickness. Seal against the casing cover sections with continuous bulbous vinyl weatherstrip interlocked within the mullion cover.

  - Interior trim:

    - Interior Trim, Closures and Angles: As detailed, of extruded shapes no less than 0.062 inch nominal wall thickness.

    - Snap Trim: Apply in full length without splices and attach with clips spaced no more than 18 inches on center. No exposed screws will be allowed on interior trim.

- ALUMINUM DOOR FINISHES

- Select appropriate finish.  Custom color baked on finishes available upon request.  Silicone Polyester is not available in all colors.  The exterior and interior of the window can be a different finish and/or color.  For dual finish, retain both finishes below and add “Exterior” and “Interior before the “Color” name.

  - Finish Options

    - Provide manufacturer’s standard acrylic or polyester, baked-on, electrostatically applied enamel coating of manufacturer’s standard color(s) [custom] as selected by the Architect, applied over manufacturer’s standard substrate preparation including cleaning, degreasing, and chromate conversion coating.

    - Finish shall meet or exceed AAMA 2603 (formerly AAMA 603). (Custom colors available when specified.)

      - Color:

      - Manufacturer’s Code:

*****OR*****

  - Provide manufacturer’s standard 2 coat Fluoropolymer 50% Kynar or Silicone Polyester, baked on, electrostatically applied enamel coating.  Color to be selected from manufacturer’s standard colors [custom non-exotic color] [custom exotic color] as selected by the Architect, applied over manufacturer’s standard substrate preparation including cleaning, degreasing, and chromate conversion coating.  Finish shall meet or exceed AAMA 2604 (formerly AAMA 605).

    - Color:

    - Manufacturer’s Code:

***** OR *****

  - Provide manufacturer’s standard 2 coat Fluoropolymer 70% Kynar baked on, electrostatically applied enamel coating.  Color to be selected from manufacturer’s standard colors [custom non-exotic color] [custom exotic color] as selected by the Architect, applied over manufacturer’s standard substrate preparation including cleaning, degreasing, and chromate conversion coating.  Finish shall meet or exceed AAMA 2605.

    - Color:

    - Manufacturer’s Code:

***** OR *****

  - Provide Class II, Clear Anodized finish to all exposed areas of aluminum windows.  Finish shall meet AAMA 611.

***** OR *****

  - Provide Class I, Clear Anodized finish to all exposed areas of aluminum windows.  Finish shall meet AAMA 611.

***** OR *****

  - Provide Class I, Color Anodized finish to all exposed areas of aluminum windows.  Finish shall meet AAMA 611. Color: [Light bronze] [Medium bronze] [Dark bronze] [Black]

PART 3 - EXECUTION

- PREPARATION

  - Existing Construction:

    - Do not remove existing door until new replacement is available and ready for immediate installation.  Do not leave any openings uncovered at end of working day, during wind-driven precipitation or during excessively cold weather.

    - Remove existing work carefully; avoid damage to existing work to remain.

  - Perform operations as necessary to prepare openings for proper installation and operation of new units.

  - Verify openings are in accordance with shop drawings and Architects Drawings.

- INSTALLATION

  - Comply with manufacturer's specifications and recommendations for installation of door units, hardware, operators and other components of work.  In no case shall attachment to structure or to components of the door system be through or affect the thermal barriers of the doors.

  - Set units plumb, level and true to line, without warp or rack of frames or sash.  Anchor securely in place.  Separate aluminum and other corrodible surfaces from sources of corrosion or electrolytic action.

  - Wedge fiberglass insulation between frames of units and construction, or between frames and blocking as applicable.  Compress fiberglass to no less than 50 percent of original thickness.

  - Set sill members and other members in bed of compound as shown, or with joint fillers or gaskets as shown, to provide weathertight construction.  Seal units following installation and as required to provide weathertight system.

- ADJUST AND CLEAN

  - Adjust operating sash and hardware to provide tight fit at contact points and at weatherstripping, for smooth operation and weathertight closure.

  - Clean aluminum surfaces promptly after installation of doors, exercising care to avoid damage to protective coatings and finishes.  Remove excess glazing and sealant compounds, dirt, and other substances.  Lubricate hardware and moving parts.

  - Clean glass promptly after installation of doors.  Remove glazing and sealant compound, dirt and other substances.

  - Existing doors and other materials removed from site become property of the Contractor who shall promptly remove same and legally dispose of at no additional cost to the Owner.

  - Comply with all applicable laws, rules and regulations.

- PROTECTION

  - Initiate all protection and other precautions required to ensure that door units will be without damage or deterioration (other than normal weathering) at time of acceptance.

  - Send to Architect, with copy to Owner, written recommendations for maintenance and protection of terrace doors following Substantial Completion of Door Contract.

END OF SECTION